# Whole-Body Motion Planning and Mobile Grasping Resource Map

This document separates the planning libraries, mobile grasping stack, and simulation environments so people can quickly find the right resource without reading repository internals first.

# At-a-glance architecture

- Fetch motion planning: use the Fetch VAMP fork for non-holonomic whole-body planning.

- Autolife motion planning: use Autolife Planning for IK, OMPL/VAMP planning, constraints, costs, and trajectory timing.

- Mobile grasping: use the Visibility-Aware Mobile Grasping repo for Fetch grasping, schedulers, perception clients, benchmarks, and ManiSkill integration.

- Simulation: use PyBullet for planner-level debugging, RLS Digital Twin for Fetch Gazebo/ROS testing, and ManiSkill + ReplicaCAD/YCB for benchmark-scale mobile grasping.

# 1. Motion planning resources

## Fetch whole-body planner

Resource: Fetch VAMP fork

Purpose: Whole-body motion planning for Fetch, including a non-holonomic base model and Fetch-specific collision resources.

### Key implementation points:

- Hybrid A* and Reeds-Shepp support non-holonomic base planning.

- Multi-layer RRT-Connect supports whole-body planning over base, torso, and arm layers.

- Fetch and Fetch extended robot resources support collision checking and validation.

- Whole-body validation, simplification, Python bindings, pointcloud collision, and PyBullet helpers are included.

- Note: I verified this fork as a vendored dependency inside the mobile grasping repo. If there is a private standalone AdaComp fork, this label can be re-linked later.

## Autolife planner

Resource: Autolife Planning

Purpose: Planning library for the Autolife robot, covering IK, motion planning, collision checking, and trajectory timing through one Python API.

### Key implementation points:

- TRAC-IK and Pink QP support unconstrained and constrained IK.

- OMPL planners use VAMP SIMD collision checking for full-body and subgroup planning.

- CasADi-compiled constraints and costs support task-space manifold and cost-aware planning.

- TOPP-RA converts geometric paths into executable trajectories.

- Autolife robot assets, RLS pointcloud assets, and the Autolife PyBullet environment are included for simulation and debugging.

# 2. Mobile grasping and receding-horizon control

## Visibility-aware mobile grasping stack

Resource: Visibility-Aware Mobile Grasping repo

Purpose: Fetch mobile grasping in cluttered, dynamic environments. The stack combines base placement, arm planning, active perception, and grasp prediction.

### Package naming:

- Repository: Visibility-Awared-Mobile-Grasping

- Python package: grasp_anywhere

- Project/environment name: mobile_grasping_in_dynamic

### Main components:

- Schedulers include default, sequential, nav-manip, nav-prepose, and closed-loop variants.

- The closed-loop baseline repeatedly runs prepose -> look-at-object -> grasp and resamples the prepose on failure.

- The ManiSkill MPC environment wrapper follows whole-body waypoint streams with velocity control and a small lookahead controller.

- Stage planners cover grasp, prepose, move, place, point, confirm, and static pointing stages.

- Perception and grasp services include GraspNet, OWL-ViT, SAM, and the perception services submodule.

- Evaluation tools include the ManiSkill benchmark runner, benchmark configs, trigger-distance sweeps, and visualization tools.

# 3. Simulation environments

## PyBullet planning environments

- Fetch: use the PyBullet helpers in the Fetch VAMP fork for lightweight planner visualization and tests.

- Autolife: use the Autolife PyBullet environment with the packaged Autolife robot assets.

- Best use: quick IK/planning debugging without ROS or Gazebo.

## Fetch Gazebo digital twin

Resource: RLS Digital Twin

Purpose: Gazebo/ROS Noetic digital twin of the Robot Living Studio with an integrated Fetch robot. Topics and control commands are intended to match the real robot.

### Key launch/resources:

- RLS environment launch file starts the Gazebo scene.

- Fetch driver workspace contains the robot drivers and control interfaces.

- Digital twin install guide covers the expected Ubuntu 20.04 and ROS Noetic setup.

## ManiSkill + ReplicaCAD/YCB

Resource: ManiSkill benchmark stack

Purpose: GPU simulation and benchmarking for Fetch mobile grasping in ReplicaCAD/YCB scenes.

### Key files/resources:

- ManiSkill MPC environment wrapper

- Default ManiSkill config

- Closed-loop ManiSkill config

- ManiSkill benchmark runner

- ReplicaCAD and YCB assets via the ManiSkill asset downloader

# 4. Recommended onboarding order

- Start with the mobile grasping README to set up the Fetch grasping stack and download required resources.

- Set up Autolife Planning separately when working on the Autolife robot or whole-body/subgroup planner API.

- Use the Fetch VAMP fork only when modifying Fetch non-holonomic whole-body planning internals.

- Use RLS Digital Twin for ROS/Gazebo Fetch testing, ManiSkill for dynamic grasping benchmarks, and PyBullet for planner-level debugging.

# 5. Open clean-up items

- Confirm whether the Fetch VAMP fork should be linked as a private standalone repo or kept as the vendored dependency link.

- Decide whether user-facing docs should preserve the repository spelling "Visibility-Awared" or use the paper title spelling "Visibility-Aware".

- Add owners/contact points for each resource if this becomes an onboarding document.

- Add a minimal "what to run first" command block after the repositories settle.

# Sources and links

- Visibility-Aware Mobile Grasping repo

- Fetch VAMP fork

- Autolife Planning repo

- Autolife Planning docs

- RLS Digital Twin repo

- Digital twin install guide

- KavrakiLab VAMP upstream

- Autolife VAMP submodule fork